Vlan configuration

Mikrotik Router

2025

subash subedi

# Create a Bridge and Add All Ports

Step 1: Select “Bridge” On that, you will find the “Bridge” option.

Step 2: Click on the “add” symbol “+”.

Step 3: In “General” you will find the option “Name” if you want to change the name, then change; otherwise, leave it. && Same line of general you will find “VLAN” option the “Enable” VLAN Filtering.

Step 4:  Now we will not change anything; select “Apply” & “OK”

Step 5:  Select “Bridge” On that you will find the “Ports” option.

Step 6: Click on the “add” symbol “+”.

Step 7: In “Interface” select the “ethernet” port, & In “Bridge” select the name of bridge In step 3, you have

create.

Step 8: Select “Apply” & “OK” Same process step for other Ethernet port also.

CMD

/interface bridge add name=[BRIDGE_NAME] vlan-filtering=yes

/

Since multiple VLANs will use the same physical router, we will bridge all interfaces and configure VLANs using bridge VLAN filtering.

/interface bridge port

add bridge=[NAME OF BRIDGE PORT] interface= ether[ether port]

/

Now, add all LAN interfaces to the bridge.

EXAMPLE

/interface bridge add name=bridge1 vlan-filtering=yes

/

/interface bridge port

add bridge=bridge1 interface=ether2

add bridge=bridge1 interface=ether3

add bridge=bridge1 interface=ether4

add bridge=bridge1 interface=ether5

/

Figure 1

# Configure VLANs

Step 1: Select “Interfaces” On that, you will find the “VLAN” option.

Step 2: Click on the “add” symbol “+”.

Step 3: In “Name” Enter your vlan name && IN “VLAN ID” Enter your vlan id number && In “Interfaces”

select the name of bridge In first, you have create.

Step 4:  Now we will not change anything; select “Apply” & “OK”

CMD

We will define VLAN IDs for each department and assign their respective interfaces.

/interface vlan

add interface=[BRIDGE_NAME] name=[VLAN_INTERFACE_NAME] vlan-id=[VLAN_NUMBER]

/

EXAMPLE

/interface vlan

add interface=bridge1 name=vlan2 vlan-id=2

add interface=bridge1 name=vlan3 vlan-id=3

add interface=bridge1 name=vlan4 vlan-id=4

add interface=bridge1 name=vlan5 vlan-id=5

/

Figure 2

# IP ASSIGN

Step 1: Select “IP” On that you will find the “Addresses” option.

Step 2: Click on the “add” symbol “+”.

Step 3: In “Address” enter the public IP & In “Network” enter the gateways of that IP.

Step 4:  In “Interface” select which interface you want to assign.

Step 5:  Select “Apply” & “OK”.

Step 2: Click on the “add” symbol “+”.

Step 6: In “Address” enter the local IP & In “Network” enter the gateways of that IP.

Step 7:  In “Interface” select which VLAN interface you want to assign.

Step 8:  Select “Apply” & “OK”

CMD

Assign IP addresses to VLAN interfaces:

/ip address

add address=[PUBLIC_IP/NETMASK] interface=[NAME_OF_ETHERNET]

add address=[LOCAL_IP_RANGE/NETMASK] interface=[NAME_OF_VLAN_INTERFACE]

/

/ip address

add address=192.168.2.1/24 interface=vlan2

add address=192.168.3.1/24 interface=vlan3

add address=192.168.4.1/24 interface=vlan4

add address=192.168.5.1/24 interface=vlan5

/

EXAMPLE

Figure 3

# Configure VLAN Filtering on the Bridge

We need to define which ports are assigned to each VLAN.

Step 1: Select “Bridge” On that, you will find the “VLANs” option.

Step 2: Click on the “add” symbol “+”.

Step 3: In “Bridge” select the name of bridge In first, you have create. && In “VLAN IDs” enter the vlan number. && In “Tagged” select the name of bridge In first, you have create. && In “Untagged” select the “ether interface”

Step 4:  Now we will not change anything; select “Apply” & “OK”

/interface bridge vlan

add bridge=[BRIDGE_NAME] vlan-ids=[VLAN_NUMBER] tagged=[BRIDGE_NAME] untagged=[PHYSICAL_PORT]

/

CMD

EXAMPLE

/interface bridge vlan

add bridge=bridge1 vlan-ids=2 tagged=bridge1 untagged=ether2

add bridge=bridge1 vlan-ids=3 tagged=bridge1 untagged=ether3

add bridge=bridge1 vlan-ids=4 tagged=bridge1 untagged=ether4

add bridge=bridge1 vlan-ids=5 tagged=bridge1 untagged=ether5

/

Figure 4

Ensure that each VLAN is untagged on the correct port and tagged on the bridge (so the router can process VLAN traffic).

# Configure Port VLAN IDs (PVIDs) on Bridge Port Interface

Set Port VLAN IDs (PVIDs) to make sure untagged traffic entering the port gets assigned the correct VLAN:

Step 1: Select “Bridge” On that, you will find the “Port” option.

Step 2: There you will find “Bridge ether Interface” && Double click on one option && In top you will find “VLAN” option select that option.

Step 3: In “PVID” you have insert the VLAN number According to interface

Step 4:  Now we will not change anything; select “Apply” & “OK”

CMD

/interface bridge port

set [find interface=ether[NUMBER]] pvid=[VLAN_NUMBER]

/

Example

/interface bridge port

set [find interface=ether2] pvid=2

set [find interface=ether3] pvid=3

set [find interface=ether4] pvid=4

set [find interface=ether5] pvid=5

/

Figure 5

# Configure DHCP for Each VLAN

Step 1: Select “IP” On that you will find the “DHCP SERVER” option.

Step 2: In “DHCP” there you will find “DHCP Setup” Click on that option. Then a popup setup will come.

Step 3: In “DHCP Server Interface” Select the “VLAN” name that you have created and “Next”.

Step 4: In “DHCP Address Space” insert the local IP and their subnet as required, or it will come automatically and “Next”.

Step 5: In “Gateway for DHCP Network” provide the local IP gateways, or it will come automatically and “Next”.

Step 6: In the “Addresses to Give Out” range of IP addresses, or it will come automatically and “Next”.

Step 7: In “DNS Servers” enter the ISP DNS that we have given Google for now; it will come automatically and “Next”.

Step 8: In “Lease Time” set as default and click “Next”

CMD

/ip pool add name=[NAME OF DHCP POOL] ranges=[RANGE OF IP]

/ip dhcp-server add name=[NAME OF DHCP SERVER] interface=[ASSIGN VLAN INTERFACE] address-pool=[NAME OF DHCP POOL] disabled=no

/ip dhcp-server network add address=[VLAN IP ADDRESS WITH SUBNET] gateway=[GATEWAY OF LOCAL IP] dns-server=[DNS OF ISP]

/ip dhcp-server enable [NAME OF DHCP SERVER]

EXAMPLE

/ip pool

add name=pool_vlan2 ranges=192.168.2.2-192.168.2.200

add name=pool_vlan3 ranges=192.168.3.2-192.168.3.200

add name=pool_vlan4 ranges=192.168.4.2-192.168.4.200

add name=pool_vlan5 ranges=192.168.5.2-192.168.5.200

/

/ip dhcp-server

add interface=vlan2 address-pool=pool_vlan2 name=dhcp_vlan2 disabled=no

add interface=vlan3 address-pool=pool_vlan3 name=dhcp_vlan3 disabled=no

add interface=vlan4 address-pool=pool_vlan4 name=dhcp_vlan4 disabled=no

add interface=vlan5 address-pool=pool_vlan5 name=dhcp_vlan5 disabled=no

/

/ip dhcp-server network

add address=192.168.2.0/24 gateway=192.168.2.1 dns-server=8.8.8.8

add address=192.168.3.0/24 gateway=192.168.3.1 dns-server=8.8.8.8

add address=192.168.4.0/24 gateway=192.168.4.1 dns-server=8.8.8.8

add address=192.168.5.0/24 gateway=192.168.5.1 dns-server=8.8.8.8

/

To assign IPs automatically to devices in each VLAN, create DHCP servers:

DHCP-VLAN2

| Figure 6 | Figure 7 |
| --- | --- |
| Figure 8 | Figure 9 |
| Figure 10 | Figure 11 |

DHCP-VLAN3

| Figure 12 | Figure 13 |
| --- | --- |
| Figure 14 | Figure 15 |
| Figure 16 | Figure 17 |

DHCP-VLAN4

| Figure 18 | Figure 19 |
| --- | --- |
| Figure 20 | Figure 21 |
| Figure 22 | Figure 23 |

DHCP-VLAN5

| Figure 24 | Figure 25 |
| --- | --- |
| Figure 26 | Figure 27 |
| Figure 28 | Figure 29 |

# Configure DNS && Route to WAN (LAN1 - ether1)

## DNS

Step 1: Select “IP” On that you will find the “DNS” option.

Step 2: In “Servers” enter the DNS of your ISP.

Step 3: Select “Apply” & “OK”

CMD

ip dns/

set servers=[Enter your ISP DNS]

/

Example

ip dns/ set servers=8.8.8.8,1.1.1.1

/

Figure 30

## Route

We assume LAN1 (ether1) is connected to the internet with PUBLIC IP.

Step 1: Select “IP” On that you will find the “Routes” option.

Step 2: Click on the “add” symbol “+”.

Step 3: In “Dst. Address” Enter “0.0.0.0/0” & In “Gateway” Public IP that is provided by ISP

Step 4:  Select “Apply” & “OK”

CMD

/ip route add dst-address=0.0.0.0/0 gateway=[Gateway_IP]

Example

/ip route add dst-address=0.0.0.0/0 gateway=192.168.1.254

Figure 31

# Enable NAT (Masquerade) for Internet Access && Block traffic from Vlan

## Enable NAT (Masquerade) for Internet Access

Step 1: Select “IP” On that you will find the “FireWall” Option.

Step 2: Click on the “add” symbol “+”.

Step 3: In “General” you will find “Chain” leave it default “srcnat”

Step 4:  Select “Action” you will find “Action” in that select “masquerade”

Step 5:  Select “Apply” & “OK”

CMD

/ip firewall nat

add chain=srcnat out-interface=[Uplink_Interface] action=masquerade

/

Example

/ip firewall nat

add chain=srcnat out-interface=ether1 action=masquerade

/

Figure 32

## Block traffic from VLAN

Step 1: Select “IP” On that you will find the “FireWall” Option.

Step 2: Select “Fliter Rules” && Click on the “add” symbol “+”.

Step 3: In “General” you will find “chain” there you have to select “forward” && In “In. Interface” and “Out. Interface” you have selected “Ether interface 1 / Uplink” also check box.

Step 4: Select “Action” you will find “Action” in that select “drop”

Step 5:  Select “Apply” & “OK”

To allow internet access but block VLAN-to-VLAN traffic, use:

CMD

/ip firewall filter

add chain=forward in-interface=!ether1 out-interface=!ether1 action=drop

Figure 33

## # Block traffic from

By default, all VLANs can talk to each other. If you want to block communication between VLANs, you need firewall rules.

Step 1: Select “IP” On that you will find the “FireWall” Option.

Step 2: Select “Fliter Rules” && Click on the “add” symbol “+”.

Step 3: In “General” you will find “chain” there you have to select “forward” && In “In. Interface” you have to select that “VLAN” for traffic pass and In “Out. Interface” you have select that “VLAN” to block the traffic.

Step 4: Select “Action” you will find “Action” in that select “drop”

Step 5:  Select “Apply” & “OK”

/ip firewall filter

add chain=forward in-interface=[source-VLAN-interface] out-interface=vlan3 action=[action] comment=[your comment]

CMD

EXAMPLE

/ip firewall filter

# Block traffic from VLAN2 to VLAN3, VLAN4, and VLAN5

add chain=forward in-interface=vlan2 out-interface=vlan3 action=drop comment="Drop VLAN2 -> VLAN3"

add chain=forward in-interface=vlan2 out-interface=vlan4 action=drop comment="Drop VLAN2 -> VLAN4"

add chain=forward in-interface=vlan2 out-interface=vlan5 action=drop comment="Drop VLAN2 -> VLAN5"

# Block traffic from VLAN3 to VLAN2, VLAN4, and VLAN5

add chain=forward in-interface=vlan3 out-interface=vlan2 action=drop comment="Drop VLAN3 -> VLAN2"

add chain=forward in-interface=vlan3 out-interface=vlan4 action=drop comment="Drop VLAN3 -> VLAN4"

add chain=forward in-interface=vlan3 out-interface=vlan5 action=drop comment="Drop VLAN3 -> VLAN5"

# Block traffic from VLAN4 to VLAN2, VLAN3, and VLAN5

add chain=forward in-interface=vlan4 out-interface=vlan2 action=drop comment="Drop VLAN4 -> VLAN2"

add chain=forward in-interface=vlan4 out-interface=vlan3 action=drop comment="Drop VLAN4 -> VLAN3"

add chain=forward in-interface=vlan4 out-interface=vlan5 action=drop comment="Drop VLAN4 -> VLAN5"

# Block traffic from VLAN5 to VLAN2, VLAN3, and VLAN4

add chain=forward in-interface=vlan5 out-interface=vlan2 action=drop comment="Drop VLAN5 -> VLAN2"

add chain=forward in-interface=vlan5 out-interface=vlan3 action=drop comment="Drop VLAN5 -> VLAN3"

add chain=forward in-interface=vlan5 out-interface=vlan4 action=drop comment="Drop VLAN5 -> VLAN4"

# Block traffic from VLAN2 to VLAN3, VLAN4, and VLAN5

Figure 34

# Block traffic from VLAN3 to VLAN2, VLAN4, and VLAN5

Figure 35

# Block traffic from VLAN4 to VLAN2, VLAN3, and VLAN5

Figure 36

# Block traffic from VLAN4 to VLAN2, VLAN3, and VLAN5

Figure 37

NOW CONNECTED TO YOUR PC and Internet connection will arrive as per VLAN interface port.